# User guide

## Receiver Sensitivity

### What is receiver sensitivity?

Receiver sensitivity describes the weakest signal that a receiver can reliably detect and decode. Think of it like the hearing threshold of a person — below a certain volume, they simply cannot make out what is being said, no matter how quiet the background noise is.

In JARS, sensitivity is entered in dBm (decibel-milliwatts). This is a standard unit used in radio engineering to express power levels on a logarithmic scale referenced to 1 milliwatt.

Because dBm uses a logarithmic scale, the numbers can seem counterintuitive at first:

| dBm value | Meaning |
| --- | --- |
| 0 dBm | 1 milliwatt — a relatively strong signal |
| -30 dBm | 0.001 milliwatt (1 microwatt) — weak |
| -90 dBm | 0.000000001 milliwatt (1 nanowatt) — very weak but typical for a sensitive receiver |
| -120 dBm | Near the noise floor — extremely sensitive receiver |

Key rule: a more negative number means a more sensitive receiver. A receiver with a sensitivity of −100 dBm can detect weaker signals than one with a sensitivity of −70 dBm.

Typical values to enter:

- A basic or short-range receiver: −70 to −80 dBm

- A good communications receiver: −90 to −100 dBm

- A high-performance or military receiver: −100 to −120 dBm.

### What causes "Communication blocked (signal too weak)"?

This message appears on the JARS Geometry Plot when the received signal from the transmitter arrives at the   receiver below its sensitivity threshold — meaning the signal is too faint for the receiver to detect at all, regardless of any jamming.

The simulation calculates the received signal power using the Free Space Path Loss (FSPL) formula, which accounts for the distance between transmitter and receiver and the operating frequency. If the result falls below the sensitivity value you entered, the receiver cannot "hear" the transmitter.

This is distinct from being jammed. The two failure modes are:

| Plot message | Cause |
| --- | --- |
| Communication blocked (signal too weak) | The transmitter's signal arrives below the receiver's sensitivity threshold — the link has failed due to range or insufficient transmit power, not jamming. |
| Communication blocked (jammed) | The signal is receivable, but the jammer's interference is strong enough to overwhelm it (J/S ratio exceeds the threshold). |

Note that when the signal is too weak, jamming success is also reported as zero — the jammer cannot disrupt a link that was never viable in the first place.

Common reasons this message appears:

- The transmitter and receiver are too far apart. Path loss increases with distance, so moving them closer (or increasing transmit power) will raise the received signal level.

- Transmit power is too low. Try increasing the transmitter power (in dBm) to compensate for the path loss.

- The sensitivity value is not negative enough. If you entered, for example, −50 when the receiver is actually capable of −95 dBm, the simulation will falsely conclude the link has failed. Double-check that the sensitivity   value reflects the real receiver's capability — remember, more negative = more sensitive.

- The frequency is very high. FSPL increases with frequency, so a higher-frequency link will suffer more path loss over the same distance.

### What is the noise floor?

Every radio receiver generates a small amount of internal thermal noise, and the environment adds further noise from atmospheric and man-made sources. The noise floor is the power level of this background noise — it sets the   absolute lower limit below which no signal can ever be recovered, regardless of how sensitive the receiver is.

It is calculated from first principles as:

Noise floor (dBm) = −174 dBm/Hz + 10·log10(bandwidth in Hz) + Noise Figure (dB)

For example, a receiver with a 25 kHz bandwidth and a 6 dB noise figure would have a noise floor around −122 dBm. No matter how sensitive you claim the receiver to be, it cannot detect signals weaker than this.

### How JARS handles this

JARS uses the sensitivity value you enter as a hard threshold — it is treated as a single number that implicitly rolls up all these physical effects (thermal noise, noise figure, required SNR for decoding). The simulation trusts whatever you type.

This means:

- If you enter a sensitivity of −150 dBm, the simulation will accept it even though no real receiver could achieve that — it is below the physical noise floor at any practical bandwidth.

- There is no internal check that your sensitivity value is physically achievable.

- Concepts like SNR (Signal-to-Noise Ratio), noise figure, and receiver bandwidth are not modelled separately.

### Practical implication

The sensitivity field is best used as a system-level parameter: enter a value that already accounts for the noise floor, noise figure, and the minimum SNR needed to decode the signal — which is exactly how manufacturers specify sensitivity in datasheets. As long as you use a realistic, datasheet-derived figure, the simulation will behave correctly.

If you want the simulation to reflect noise floor effects more rigorously, the model would need to be extended with receiver bandwidth and noise figure inputs, deriving the sensitivity threshold from those rather than accepting it   directly.

### Communication blocked (jammed)

This status appears on the geometry plot when the jammer has successfully disrupted communications. It means:

- The Tx signal was strong enough to reach the Rx (above receiver sensitivity)

- But the jammer signal overpowered it — the J/S ratio exceeded the threshold.

This is distinct from "Communication blocked (signal too weak)", where the Tx signal couldn't reach the Rx regardless of jamming.

The geometry plot annotates two link paths:

| Annotation | What it shows |
| --- | --- |
| Tx → Rx x.x dBm | Power received at Rx from the transmitter, calculated using Free Space Path Loss from the Tx position. |
| Jam → Rx x.x dBm | Power received at Rx from the jammer, calculated using Free Space Path Loss from the jammer position. |

Both use the same formula:

Received power (dBm) = Transmit power (dBm) − FSPL

where FSPL = 20·log₁₀(distance_km) + 20·log₁₀(frequency_MHz) + 32.44

The J/S ratio is simply:

J/S (dB) = Jam→Rx (dBm) − Tx→Rx (dBm)

When J/S exceeds the threshold, jamming is successful. So, the user can read directly from the two annotated dBm values:

- If Jam→Rx dBm is significantly higher than Tx→Rx dBm, the jammer is winning — the J/S is positive and large.

- The larger the gap between Jam→Rx and Tx→Rx, the more effective the jamming.

## Dev notes

How appropriate is it to use a normal distribution for jammer power and for x, y and z coordinates for jammer position?

This is a good question to examine carefully. The answer differs between power and position.

### Jammer Power — Normal in dBm

Why it's a reasonable choice: Power in dBm is already on a log scale. A normal distribution in dBm is equivalent to a log-normal distribution in   linear power (watts). Log-normal is well-justified for RF power uncertainty because real-world power variation typically arises from multiplicative effects — component tolerances, cable losses, gain variations — which become    additive in dB space. So, the current model is more physically grounded than it might first appear.

Where it falls short:

- Unbounded: a normal distribution allows the jammer to produce arbitrarily low or arbitrarily high power, including values below its minimum operating level or above its rated maximum. For small std values this rarely      matters in practice, but it's physically incorrect.

- Assumes symmetric uncertainty: in reality, a jammer running near its rated maximum can't exceed it, creating a     hard upper bound that the normal distribution doesn't respect.

Better alternative: A truncated normal bounded by the jammer's minimum and maximum power ratings would be more   physically correct. For most practical std values, the difference is small — but it becomes significant if the std   is large relative to the distance from the bounds.

---

### Jammer Position — independent normals in x, y, z

This is where the assumptions are more questionable.

Independence of x, y, z: The model treats the three coordinates as completely independent. This is only valid if position uncertainty is aligned with the coordinate axes. In practice:

- A jammer operating from a vehicle follows roads — uncertainty is primarily along the road direction, not axis-aligned.

- A jammer at a known observation post has correlated positional uncertainty based on terrain.

- Modelling these correctly requires a multivariate normal with a covariance matrix, or a different distribution entirely.

The z-coordinate is particularly problematic.  For a ground-based jammer, altitude is tightly constrained by terrain. A normal distribution centred at z=0 with a   std of, say, 20m will regularly sample negative z values — placing the jammer underground. For an airborne platform, altitude uncertainty has a very different character (constrained by flight level, terrain avoidance).

### Operational vs. measurement uncertainty

The normal distribution is a reasonable model for measurement uncertainty — where you know roughly where the jammer is but have imprecise GPS or intelligence. It is a poor model for operational uncertainty — where you're asking, "Where might the jammer choose to deploy within a patrol area?" In that case, a uniform distribution over a geographic area is more appropriate, since all accessible points within the area are roughly equally plausible.

### Summary

| Parameter | Normal appropriate | Key limitation | Better alternative |
| --- | --- | --- | --- |
| Jammer power | Yes, reasonable | Unbounded – ignores min/max ratings | Truncated normal |
| Jammer x/y position | Depends on scenario | Independence assumption; axis-aligned only | Multivariate normal or uniform over area |
| Jammer z position | Rarely | Allows underground / implausible altitudes | Truncated normal with terrain bounds |

### Practical implication for the tool

The current choice of normal distributions is a pragmatic starting point and produces sensible results when std values are modest relative to mean distances. The most valuable addition would be offering uniform distribution as an alternative for x/y position — this better represents the "area search" scenario where you know the jammer is somewhere within a defined zone but have no reason to favour the centre over the edges. This is a common operational planning scenario.